Question Bank

Data Communication and Security

Unit 1: Introduction and Physical Layer (15 Sessions)

Q1. Explain the different types of computer networks with suitable examples.

Q2. Explain the different types of network topologies with suitable examples.

Q3. Give an overview of computer networks. Explain their basic components, advantages, and applications with suitable examples.

Q4. Explain the OSI (Open Systems Interconnection) reference model. Describe its seven layers and their functions with suitable examples.

Q5. Explain the TCP/IP model. Describe its four layers, their functions, and give suitable examples.

Q6. Compare the OSI and TCP/IP reference models.

Q7. Explain guided transmission media. Describe its types, advantages, disadvantages, and applications with suitable examples.

Q8. Explain Unguided transmission media. Describe its types, advantages, disadvantages, and applications with suitable examples.

Q9. Differentiate between guided and unguided transmission media.

Q10. Explain the concept of Analog and digital communication with examples. Discuss their advantages and limitations.

Q11. Describe Shannon’s Capacity Theorem. How does it influence the design of modern communication systems?

Q12. Explain the process of encoding and modulation in digital communication. Illustrate with a neat diagram.

Q13. Explain the concept of Multiplexing in data communication. Detail the working principle of Time Division Multiplexing (TDM), differentiate between Synchronous TDM and Asynchronous (Statistical) TDM with neat block diagrams.

Q14. Explain switching techniques used in communication networks. Highlight their role in efficient resource utilization.

Q15. What are three different transmission impairments? Explain different types of noise.

Q16. Explain Bit stuffing and Byte stuffing with appropriate example. Which method is good? Why?

Q16.Describe CSMA protocols. Compare persistent and non-persistent CSMA in terms of performance.

Q17. What is the maximum bit rate of a noiseless channel with a bandwidth of 5000 Hz transmitting a signal with two signal levels.

Solution:- We apply the Nyquist Bit Rate formula:

Bit Rate = 2 x Bandwidth x log2 L    [C=2 *B*Log(2) L]

Where C= Bit Rate per second, B=Bandwidth, L=signal Level

Given :- B=5000Hz, L=2 use above formula

C=2*5000* log2   *2

C=10000 * 1

C=10000 BPS

Q18. A noiseless channel of bandwidth B = 5 kHz must support a bit rate of C= 90 kbps. What is the required number of signal levels L?

Solution:-

We use the Nyquist Bit Rate formula:

C=2B⋅log2L

Given:

- C=92,000 bps

- B=5,000 Hz

Step 1: Rearrange for L

log2L=C2B

Step 2: Substitute values

log2L=92,0002⋅5000=92,00010,000=9.2

Step 3: Solve for L

L=29.2≈600

Q19 A noiseless channel with a bit rate of 96 kbps and 4 signal levels — determine the required bandwidth.

Solution:- We use the Nyquist Bit Rate formula:

C=2B⋅log2L

Given:

- C=96,000 bps

- L=4signal levels → log24=2

Step 1: Substitute values

96,000=2B⋅2

96,000=4B

Step 2: Solve for B

B=96,0004=24,000 Hz

The required bandwidth is 24 kHz.

Q20. A noisy channel has a bandwidth of 4 kHz and a signal-to-noise ratio of 30 dB. Calculate the maximum data rate of the channel using Shannon capacity theorem.

Solution:- We apply the Shannon Capacity Theorem:

C=B⋅log21+SN

Given:

- Bandwidth B=4 kHz=4000 Hz

- Signal-to-Noise Ratio (SNR) = 30 dB

For calculating SN there is formula

30=10 * log10 (S/N)

Log10(S/N)=30/10

Log10(S/N)=3

S/N=103

S/N=1000 (signal is 1000× more powerful than the noise, which indicates a very clean communication channel.)

Now put the value of S/N in below formula

C=B⋅log21+SN

C= 4000 * log2 (1+1000)

C= 4000 * log2 (1001)

log21001means the logarithm of 1001 to the base 2.

log21001=ln⁡1001ln⁡2

ln⁡1001≈6.908 and ln⁡2≈0.693

log21001≈6.9080.693≈9.97

So, log21001≈9.97.

C= 4000 * 9.97

C=39880 BPS

C=39.9KBPS

Encode the bit stream 110110010 using Unipolar NRZ. Draw the digital waveform.

Encode the bit stream 101101010 using Polar NRZ-L. Present the waveform.

Encode the bit stream 101101010 using Polar NRZ-I (assume initial level is Low). Draw the waveform.

Encode the bit stream 1011011010 using Return-to-Zero (RZ). Present the waveform.

Encode the bit stream 101110010 using Manchester encoding. Draw the waveform.

Encode the bit stream 101101010 using Differential Manchester encoding (assume initial level is Low). Present the waveform.

Unit 2: Data Link Layer (15 Sessions)

Q1. Define character stuffing in computer networks. Explain how it is used in character-oriented framing with a suitable example.

Q2. Define bit stuffing in computer networks. Explain how it is used in bits-oriented framing with a suitable example.

Q3. Describe Vertical Redundancy Check (VRC), Longitudinal Redundancy Check (LRC with examples.

Q4. The sender transmits the following byte code to the receiver. Using the Vertical Redundancy Check (VRC) method, demonstrate that an error has occurred in the received byte code.

| Sr No | Sender side | Receiver side |
| --- | --- | --- |
| 1 | 1 0 1 1 1 0 1 0 | 1 0 1 1 1 0 1 1 |
| 2 | 1 1 1 1 1 0 1 0 | 1 1 0 1 1 0 1 0 |
| 3 | 1 1 0 0 1 0 1 0 | 1 1 1 0 1 0 1 1 |
| 4 | 0 1 0 0 1 0 1 0 | 0 1 0 0 1 0 1 0 |

Q5. The sender transmits the following byte code to the receiver. Using the Longitudinal Redundancy Check (LRC) method, demonstrate that an error has occurred in the received byte code.

| Sr No | Sender side | Receiver side |
| --- | --- | --- |
| 1 | 1 0 1 1 1 0 1 0 1 1 1 1 1 0 1 0 | 1 0 1 1 1 0 1 0 1 1 0 1 1 0 1 0 |
| 2 | 1 1 0 0 1 0 1 0 0 1 0 1 0 1 0 1 | 1 1 0 0 1 0 1 0 0 1 0 1 0 1 1 1 |
| 3 | 1 1 0 1 1 0 1 0 0 1 0 1 1 1 0 1 | 1 1 0 1 1 0 1 0 0 1 0 1 1 1 0 1 |

Q6. Define Hamming Code. Explain its role in error detection and correction. Illustrate with a suitable example how single-bit errors can be detected and corrected using Hamming Code.

Q7. The given data bits are 1010010101. Construct the Hamming Codeword using even parity.

Q8. The received Hamming codeword is 01100101001011. Identify the position of the error using even parity checks and correct the codeword accordingly.

Q9. Explain the concept of flow control at the data link layer. Explain the working of Stop-and-Wait ARQ protocol.

Q10. Describe the Sliding Window protocol. How does it improve efficiency compared to Stop-and-Wait?

Q11. Explain the Go-Back-N ARQ technique. Illustrate its operation with an example and discuss its advantages.

Q12. Describe Selective-Reject ARQ. Compare it with Go-Back-N ARQ in terms of efficiency and complexity.

Q13. Explain the High-Level Data Link Control (HDLC) protocol. Explain its frame types (I-frame, S-frame, U-frame) and explain the significance of every field in the HDLC frame structure with a neat diagram.

Q14. Discuss channel allocation methods in computer networks. Explain fixed and dynamic allocation with examples.

Q15. Discuss the Channel Allocation Problem in Medium Access Control (MAC). Explain Static vs. Dynamic channel allocation techniques. Explain the working principles, collision handling, and throughput performance of Pure ALOHA, Slotted ALOHA.

Q 16. A sender transmits the 4-bit data 1011. Construct the Hamming Code, show parity bit placement, and write the final codeword.

| 17. A sender has four 8-bit data words: 110010100, 101001010, 111100100 and 001101100. Calculate the 8-bit checksum using 1's complement addition and write the transmitted data with checksum. - A sender transmits the 4-bit data 1011. Construct the Hamming Code, show parity bit placement, and write the final codeword. - A 7-bit Hamming codeword 1011011 is received. Detect and correct the error (if any) using the Hamming method. Show all steps. - Encode the dataword 10010110 using CRC with generator polynomial x3+x2+1. Perform the division and determine the transmitted codeword. - At the receiver side, the codeword 10010110101 is received. Using the same generator polynomial x3+x2+1, check whether the data should be accepted or rejected.. - A sender transmits the message 11001100. Compute the checksum at the sender side and verify it at the receiver side. Show all steps. - A message 10101010 is transmitted using Vertical Redundancy Check (VRC). Show how errors can be detected at the receiver. - Encode the message 10101010 using Longitudinal Redundancy Check (LRC). Construct the matrix and show how errors are detected. - Compare the effectiveness of VRC and LRC in detecting single-bit and burst errors. - A sender transmits the data 11100011. Construct the Hamming Code and demonstrate how a single-bit error is corrected at the receiver. |  |
| --- | --- |